SIH 2026  ·  PS 26052 (DRDO)  ·  Team ZEIT v.1.0 (HW2026106)

ZEIT Hardware

Every part, every port, every wire, and how the headset is built

| Version | 2.0 — draft for team review |
| --- | --- |
| Date | 17 September 2026 |
| Status | Designed, not built. Every claim carries a status tag |
| Companion document | ZEIT System Architecture (step IDs A1–A11, R1–R5, B1–B11, T1–T8) |
| Supersedes | Architecture v1 (Raspberry Pi 5 design) in handbook/index.html and docs/01-system/architecture.md |

Five blank revision pages are provided at the end of this document for your changes.

# Contents

1. How to use this document

2. Summary

3. Bill of materials (H1–H24)

4. Microphone map

5. Preamp / bias board

6. Board port assignments

7. Cables and connector pinouts

8. Power and grounding

9. Building the headset

10. Tests after building

11. Hardware ↔ neural network compatibility

12. Budget

13. Gates and checklists

14. Decisions log

Revision notes (5 pages)

# 1. How to use this document

Each part has an ID (H1–H24). The "Used by" column names the architecture steps. If a part changes, check those steps; if a step changes, check the parts it names. Prices are September 2026 estimates including GST; confirm with quotations.

## Status tags

| Tag | Meaning |
| --- | --- |
| VERIFIED | Checked against a published source (datasheet, standard, manufacturer page) |
| MEASURED | We ran it ourselves and have the number |
| ESTIMATE | Engineering arithmetic; not yet tested |
| UNVERIFIED | A risk. Must be checked before relying on it |

# 2. Summary

IN PLAIN WORDS A modified 3M PELTOR earmuff with six microphones and two speakers. One ADAU1787 DSP protects both ears; a Jetson runs the AI that cleans both voices; a walkie-talkie pair stands in for the military radio.

| Headset | 3M PELTOR Optime III, modified: 6 mics, 2 speakers |
| --- | --- |
| Lane A | EVAL-ADAU1787Z (one board, both ears) + STM32 Nucleo-H743ZI2 |
| Lane B | Jetson Orin Nano Super + Behringer UMC404HD |
| Radio | PMR446 pair (WPC ETA), isolated interface |
| Must-buy total | ≈ ₹1,11,550 – ₹1,37,250 (ESTIMATE) |
| Budget requested | ₹1,25,000 |

HEADSET: Mic 1, 2L, 2R, 3L, 3R ─ 8-core GX16 ─┐   speakers ◄─ 4-core GX16 ◄─ H13 amp ◄─┬─ ADAU DAC L/R

Mic 4 throat ─ 3.5 mm ───────────────┼─► PREAMP H6 ─┬─► UMC in 1–3 ─USB─► JETSON └─ UMC out 2 (radio voice)

PTT button ─ 3.5 mm ─► STM32 input   │              └─► ADAU ADC 1–4          │ UART

│   radio RX ─► H19 ─► UMC in 4          ▼

│   UMC out 1 ─► H19 ─► radio mic    STM32 ─I2C/TDM via H12a/b─► ADAU

# 3. Bill of materials (H1–H24)

| ID | Part | Key spec | Qty | Est. ₹ | Used by | Buy |
| --- | --- | --- | --- | --- | --- | --- |
| Headset and sensors |  |  |  |  |  |  |
| H1 | 3M PELTOR Optime III H540A earmuff (alt. X5A) | Passive, SNR 35 dB (X5A 37) — VERIFIED; double casing | 1 | 2,000–5,000 | Protection; shell | Must |
| H1-P | Basic earmuff | Practice | 1 | ~500 | Fitting practice | Must |
| H1-A | 40 mm drivers, 32 Ω | 2 fitted + 2 spare | 4 | 400–1,000 | Speakers (A10–A11, R5) | Must |
| H1-B | Gooseneck electret boom mic | 2-wire electret | 1 | 300–800 | Mic 1 (B1, B7) | Must |
| H1-C | Acrylic / plastic sheet | Speaker rings | 1 | 150–300 | Speaker mounts | Must |
| H1-D | Acoustic foam sheet | Replaces cup foam | 1 | 200–400 | Damping, seal | Must |
| H1-E | Boom clamp | Left cup | 1 | 100–300 | Mic 1 | Must |
| H1-F | Acoustic mesh | Covers inside mics | 1 | 50–150 | Mic 3L, 3R | Must |
| H1-G | PTT button + 3.5 mm cable | Momentary | 1 | 100–300 | B11 | Must |
| H2 | Infineon IM73A135 breakout | AOP 135 dB, SNR 73 dB, VDD 2.3–3.0 V — VERIFIED | 2 | 1,600–5,000 | Mic 2L, 2R (A1–A6, B1–B7) | Must (Gate 3) |
| H3 | SparkFun ICS-40180 MEMS breakout (BOB-18011) | 1.5–3.6 V, SNR 65, op-amp output — VERIFIED; check stock | 3 | 750–1,500 | Mic 3L, 3R + spare | Must |
| H4 | Throat mic, 3.5 mm | Electret laryngophone | 1 | 1,500–3,000 | Mic 4 (B1, B7, B9) | Must |
| H5 | Foam windshields | Mic foams | 3 | 300–700 | Mic 2L, 2R, 1 | Must |
| H6 | Preamp / bias board | 6 in, 7 out, L/R switch | 1 | 3,000–3,500 | All mics | Must (build) |
| H7 | Foam earplugs | Rated | 1 box | ~300 | Double protection | Must |
| H8 | Cables, GX16, sealing | 8-core + 4-core shielded, GX16-8 + GX16-4, grommets, silicone | 1 set | 1,300–2,200 | Headset connections | Must |
| Lane A electronics |  |  |  |  |  |  |
| H9 | EVAL-ADAU1787Z | 4 ADC, 2 DAC, FastDSP 768 kHz — VERIFIED | 1 | ~32,000 | A1–A11 | Must (Gates 1, 2) |
| H10 | USBi programmer | SigmaStudio interface | 0–1 | 0 / ~9,000 | Programming H9 | Only if needed |
| H11 | STM32 Nucleo-H743ZI2 | UART, I²C, SAI, ADC, button | 1 | ~3,500 | Boot, modes, A8, PTT, volume | Must |
| H12a | PCA9306 I²C level shifter | 1.8 ↔ 3.3 V, made for I²C | 1 | 100–200 | H11 ↔ H9 control | Must unless on-board |
| H12b | TXB0104 push-pull shifter | 1.8 → 3.3 V, BCLK / FSYNC / SDATA | 1 | 100–200 | H9 → H11 (A8) | Phase 2 |
| H13 | TPA6130A2 amp | 2.5–5.5 V, run at 3.3 V; 32 Ω; I²C volume | 1 | 1,000–2,000 | Sums DSP + radio; drives speakers | Must |
| Lane B electronics |  |  |  |  |  |  |
| H14 | Behringer UMC404HD | 4 in, 24-bit, USB class-compliant | 1 | ~15,000 | B1, B11, R1, R5 | Must |
| H15 | Jetson Orin Nano Super Dev Kit | 6× A78AE, Ampere 1024 CUDA, 8 GB, 67 TOPS, 7–25 W — VERIFIED | 1 | 37,200–45,200 | B1–B11, R1–R5 | Must (Gate 6) |
| H16 | microSD 128 GB A2 | Boot + storage | 1 | ~1,500 | H15 | Must |
| H17 | Hum fix | Grounding → ground-loop isolators → high-speed USB isolator (not ADuM3160) | 0–1 | 0 / check | B1 | Only if hum |
| Communication unit |  |  |  |  |  |  |
| H18 | PMR446 pair | WPC ETA; K-type jack — rule VERIFIED | 1 pair | 2,500–6,000 | B11, R1 | Must (Gate 5) |
| H19 | Radio interface parts | 2 transformers, PC817, resistors, K-plug cable | 1 set | 600–1,200 | B11, R1 | Must |
| Power and test |  |  |  |  |  |  |
| H20 | Clean 5 V supply | Separate from Jetson | 1 | ~1,000 | H6, H9, H11 (H13 from the Nucleo 3.3 V) | Must |
| H21 | PD bank + 19 V trigger | Field power | 0–1 | 0 / ~5,000 | H15 | Outdoor demo only |
| H22 | Mannequin + ear mic | Headset must fit | 1 | ~2,000 | Calibration, Lane A tests | Must |
| H23 | Sound level meter | UNI-T UT353BT class | 1 | ~2,500 | Calibration, safety | Must |
| H24 | Calibrator 94 dB / 1 kHz | — | 0–1 | 0 / 6,000–12,000 | Absolute dB SPL | Optional |

| Group | Est. ₹ |
| --- | --- |
| Headset and sensors | 12,550 – 24,950 |
| Lane A electronics | 36,700 – 37,900 |
| Lane B electronics | 53,700 – 61,700 |
| Communication unit | 3,100 – 7,200 |
| Power and test | 5,500 |
| Must-buy total | 1,11,550 – 1,37,250 |
| Conditional (H10, H17, H21) | up to ~14,000 + isolator if needed |
| Optional (H24) | 6,000 – 12,000 |

Tools if the lab lacks them: mini drill + step bit, temperature-controlled soldering iron, multimeter, glue gun (≈ ₹2,600–4,800).

# 4. Microphone map

| Mic | Position | Part | Preamp in | Preamp out | Destination |
| --- | --- | --- | --- | --- | --- |
| Mic 1 | Boom, left | H1-B | IN1 | OUT1 | UMC in 1 → AI input 0 |
| Mic 2L | Outside left | H2 | IN2 | OUT5 (+ OUT2 if switch = L) | ADAU ADC 1 |
| Mic 2R | Outside right | H2 | IN3 | OUT6 (+ OUT2 if switch = R) | ADAU ADC 2 + UMC in 2 → AI input 1 |
| Mic 3L | Inside left | H3 | IN4 | OUT7 | ADAU ADC 3 |
| Mic 3R | Inside right | H3 | IN5 | OUT8 | ADAU ADC 4 |
| Mic 4 | Throat | H4 | IN6 | OUT3 | UMC in 3 → AI input 2 |

# 5. Preamp / bias board (H6)

| Function | Design |
| --- | --- |
| Inputs | 6: IN1 boom (electret), IN2–IN5 MEMS (3-wire), IN6 throat (electret) |
| Bias | Electret 2–5 V via resistor (IN1, IN6); MEMS on a shared 2.8 V LDO (IM73A135 max 3.0 V — never 3.3 V) |
| Gain | Per channel, OPA1678 on 5 V (2.5 V mid-rail, ≈ 1.2 Vrms max); a shout ≈ −12 dBFS at the converters |
| ADAU side | AC-coupled, 1 kΩ series, gain below the ADAU full scale (value from the datasheet) |
| UMC side | TRS line inputs, phantom power OFF |
| Outputs | 7 buffered: OUT1–OUT3 → UMC in 1–3; OUT5–OUT8 → ADAU ADC 1–4 (OUT4 unused) |
| Handedness switch | SPDT: Mic 2R (right-handed, default) or Mic 2L (left-handed) to OUT2 |
| Supply | Clean 5 V (H20); star ground on this board |

# 6. Board port assignments

## EVAL-ADAU1787Z (H9)

| Port | Signal | Notes |
| --- | --- | --- |
| ADC 1–4 | Mic 2L, Mic 2R, Mic 3L, Mic 3R | From H6 OUT5–OUT8 |
| DAC L / R | → H13 (summing resistors) | 79 dB share |
| I²C | ← H11 via H12a (PCA9306) | Control, program load |
| Serial (I²S/TDM) | → H11 SAI via H12b (TXB0104) | Adaptive ANC phase only |
| USB | Laptop, SigmaStudio | Development; power |

## Behringer UMC404HD (H14)

| Port | Signal | Notes |
| --- | --- | --- |
| In 1 / 2 / 3 (TRS) | Mic 1 / reference / Mic 4 | AI inputs 0 / 1 / 2; gains set once and logged |
| In 4 (TRS) | Radio RX (field) · mannequin ear mic (lab) | Swap the cable |
| Out 1 | → H19 → radio mic in | Outgoing voice |
| Out 2 | → H13 (summing resistors) | Incoming voice |
| 48 V phantom | OFF | H6 powers the mics |

## Jetson Orin Nano Super (H15)

| Port | Use | Notes |
| --- | --- | --- |
| USB | UMC404HD | Audio class |
| 40-pin UART | STM32 link | 3.3 V; assign pins from NVIDIA's header pinout |
| 40-pin GPIO | Not used for PTT | Header pins go through TXB0108 shifters (≈ 2–3 mA) — cannot drive the optocoupler |
| DisplayPort | Monitor | Not HDMI |
| microSD / DC | H16 / 19 V adapter (H21 in field) | — |

## STM32 Nucleo-H743ZI2 (H11)

| Port | Use |
| --- | --- |
| UART | Jetson link (protocol in the architecture document) |
| I²C | ADAU1787 (via H12a) and TPA6130A2 volume |
| SAI | Mic data from the ADAU for A8 (via H12b) |
| GPIO in / out | PTT button (pull-up) / PC817 LED via 330 Ω (≈ 6 mA) |
| 3V3 pin | Supplies H13 |
| User button | ANC on / PASSIVE |
| USB | Power (H20) and flashing |

## Radio interface (H19)

| Path | Parts | Notes |
| --- | --- | --- |
| UMC out 1 → radio mic | Divider → transformer 1 → 1–4.7 µF DC-block cap | The radio puts bias on its mic line and keys PTT when it is grounded; the cap stops the transformer shorting it |
| STM32 GPIO → radio PTT | PC817 optocoupler | Isolated; check which K-plug contact keys PTT |
| Radio speaker → UMC in 4 | Divider → transformer 2 | Isolated; both primary ends float (speaker amp may be bridged) |

# 7. Cables and connector pinouts

## Microphone cable — 8-core shielded, GX16 8-pin

| Pin | Signal |
| --- | --- |
| 1 | MEMS supply (VDD) |
| 2 | Microphone ground |
| 3 | Mic 1 (electret signal + bias) |
| 4 | Mic 2L |
| 5 | Mic 2R |
| 6 | Mic 3L |
| 7 | Mic 3R |
| 8 | Shield (grounded at the preamp end only) |

## Speaker cable — 4-core shielded, GX16 4-pin

| Pin | Signal |
| --- | --- |
| 1 | Left + |
| 2 | Left − |
| 3 | Right + |
| 4 | Right − |

Mics and speakers in separate cables so speaker current cannot couple into the mics. Mic 4 and the PTT button have their own 3.5 mm leads. Label every wire at both ends.

# 8. Power and grounding

| Board | Supply | Why |
| --- | --- | --- |
| Jetson (H15) | Own 19 V adapter; H21 in the field | Keeps switching noise from the mics |
| UMC404HD (H14) | USB bus power from the Jetson | Phantom off keeps current low |
| H6, H9, H11, H13 | Separate clean 5 V (H20) | Mic noise floor depends on it |
| Radio (H18) | Own batteries | Isolated by H19 |

- Star ground at the preamp board; shields grounded at one end; short leads.

- Hum: fix grounding first; then ground-loop isolators on analog lines; a high-speed USB isolator last. ADuM3160 boards are full-speed only and will not carry the UMC404HD.

# 9. Building the headset

IN PLAIN WORDS Buy a ready earmuff and add the parts: speakers inside both cups, an inside mic next to each ear, an outside mic on each cup, a boom mic on the left. The throat mic is worn separately on the neck. Practise every drilling step on the cheap earmuff first.

- Open the cups. Remove cushions and inner foam; note how they seal.

- Speakers (H1-A, H1-C). Ring per cup holding a 40 mm driver facing the ear; seal with silicone. The cup must stay airtight — that is the passive protection.

- Inside mics (H3, H1-F). One per cup, between speaker and ear, close to the ear canal, facing the ear, on a foam pad; mesh over. Do not touch the speaker cone.

- Outside mics (H2, H5). One small hole through both casing layers per cup (step bit, slow, masking tape). Board facing out, recessed; seal both layers; windshield. Mic 2R on the right cup (away from the boom), Mic 2L on the left.

- Boom (H1-B, H1-E, H5). Clamp the gooseneck to the left cup; tip 2–5 cm from the corner of the mouth; windshield.

- Wiring (H8). One grommeted exit per cup; left-cup wires along the headband; 8-core mics, 4-core speakers, GX16 per section 7; strain relief.

- Throat mic (H4), PTT (H1-G). On their own leads.

- Close up. New foam (H1-D), cushions back; check the seal on a head.

# 10. Tests after building

| Test | How | Pass |
| --- | --- | --- |
| Wiring | Multimeter end to end and between adjacent pins | Continuous, no shorts |
| Every mic alive | Record each channel while tapping near it | Each mic on its own channel |
| Speakers | Slow sweep into each ear | Clean, no rattle, L/R separate |
| Seal | Loud noise outside; outside vs inside mic, system off | Inside clearly quieter, consistent with the rating |
| Voice leak | Speak in a quiet room; Mic 1 vs reference | Reference ≥ 25 dB quieter; otherwise move Mic 2R further back |
| Throat mic | Speak with loud noise playing | Voice clear, background low |

Then calibration (architecture document, section 6).

# 11. Hardware ↔ neural network compatibility

| Link | Hardware | Model / training | Status |
| --- | --- | --- | --- |
| Inputs and order | UMC in 1 / 2 / 3 | Inputs 0 / 1 / 2 | Locked; label cables |
| Sample rate | 48 kHz | 16 kHz | Resampler MEASURED |
| Frame / hop | 768 @ 48 kHz | 256 @ 16 kHz | Parity MEASURED |
| Reference overload | Clips above 135 dB | Clipping augmentation | Built |
| Gains | H6 per channel | Random per-channel gain | Input 1 built; input 2 to add |
| Voice in reference | Head between boom and Mic 2R | Leak −45 … −25 dB | Built; voice-leak test |
| Throat channel | H4 | Real + simulated + dropout | To implement |
| Handedness | Switch on H6 | Same model either way | No retraining |
| TensorRT | H15 | GRU layers run in FP32 / FP16, not INT8 (VERIFIED, NVIDIA docs) | GRUs stay FP16; INT8 only for convolutions; engine build on the Jetson still to do |

AUDIT A link-by-link compatibility audit (docs/01-system/compatibility-audit.md) found and fixed: PTT moved from the Jetson to the STM32; PCA9306 + TXB0104 instead of one 8-channel shifter; MEMS on 2.8 V (IM73A135 max 3.0 V); H3 named (ICS-40180); TPA6130A2 on 3.3 V; DC-block capacitor on the radio mic line; AC coupling into the 1.8 V ADAU; UART protocol defined; data loader and live runner moved to 3 inputs.

# 12. Budget

|  | Est. ₹ |
| --- | --- |
| Must-buy total | 1,11,550 – 1,37,250 |
| Budget requested | 1,25,000 |
| What fixes the figure | Gate 1 (USBi), Gate 4 (earmuff price), Gate 6 (Jetson quote) |

# 13. Gates and checklists

## Before ordering

- UG-1532: built-in USB? I/O voltage and level shifting? self-boot? serial pins?  (H9–H12)

- ADAU1787 FastDSP capacity for two ANC paths  (H9)

- IM73A135 as a ready breakout in India (else another ≥ 130 dB analog MEMS)  (H2)

- Earmuff price; cup depth  (H1)

- PMR446: WPC ETA, K-type jack, pinout  (H18, H19)

- Written Jetson quotation  (H15)

## After arrival

- Record 4 channels from the UMC404HD on the Jetson (arecord).

- Build the 3-input model as a TensorRT engine; frame time < 16 ms.

- Build the headset; run the section 10 tests.

- SigmaStudio: hear-through + limiters, then fixed ANC (both ears), then adaptation.

- Calibrate on the mannequin.

- Hum check; H17 only if needed.

# 14. Decisions log

| Question | Decision | Why |
| --- | --- | --- |
| Raspberry Pi or Jetson? | Jetson Orin Nano Super | PS names Jetson and TensorRT |
| Aviation headset or earmuff? | Earmuff, modified | No built-in electronics; more protection; cheaper |
| Bose A30? | No | Active ANC of its own; ≈ ₹1 lakh+ |
| How many mics? | Six; three into the AI | Outside + inside per ear; voice, reference, throat for the AI |
| One or two ADAU1787? | One | 4 in / 2 out fits stereo hybrid ANC once the radio joins at the amp |
| Process incoming radio voice? | Yes, on the Jetson | Intelligibility applies to what the wearer hears |
| AI reference mic? | Mic 2R (2L for left-handed) | Far side of the head from the mouth |
| Fixed or adaptive ANC first? | Fixed first | Lower risk |
| USB isolator? | Only if hum; not ADuM3160 | ADuM3160 is full-speed only |
| PTT or VOX? | PTT | VOX would transmit gunshots |

Revision notes — page 1 of 5

ZEIT Hardware · write changes here · reference step / item IDs (e.g. A4, B7, H9) so both documents stay aligned

Revision notes — page 2 of 5

ZEIT Hardware · write changes here · reference step / item IDs (e.g. A4, B7, H9) so both documents stay aligned

Revision notes — page 3 of 5

ZEIT Hardware · write changes here · reference step / item IDs (e.g. A4, B7, H9) so both documents stay aligned

Revision notes — page 4 of 5

ZEIT Hardware · write changes here · reference step / item IDs (e.g. A4, B7, H9) so both documents stay aligned

Revision notes — page 5 of 5

ZEIT Hardware · write changes here · reference step / item IDs (e.g. A4, B7, H9) so both documents stay aligned